Ques 1. A cylindrical bar is 20mm diameter and 1000 mm long. During a tensile test it is found that the longitudinal strain in 4 times the lateral strain. Calculate the modulus of rigidity(C) and the bulk modulus, if its elastic modulus is 1x105 N/mm2 . Find the change in volume when the bar is subjected to a hydrostatic pressure of 100 N/mm2

Solution: d=20mm;

l=1000mm

Longitudinal strain = 4 times lateral strain

E = 1x105 N/mm2 , p = 100 N/mm2

Rigidity modulus C=?

Bulk modulus K=?

δV = ?

Poisson’s ratio = μ = 𝑙𝑎𝑡𝑒𝑟𝑎𝑙 𝑠𝑡𝑟𝑎𝑖𝑛 𝑙𝑜𝑛𝑔𝑖𝑡𝑢𝑑𝑖𝑛𝑎𝑙 𝑠𝑡𝑟𝑎𝑖𝑛 = 1/4 = 0.25

E = 2C (1+ μ)

C = E/2(1+ μ)

= 1*105 / (2*(1+0.25))

= 4 ∗ 104 𝑁/𝑚𝑚2

E = 3K (1-2 μ)

K = E/ 3 (1-2 μ)

= 1∗105 / (3*( 1−2∗0.25)) = 2/3 ∗ 105 𝑁/𝑚𝑚2

Volumetric strain = 𝑒𝑣 = 𝑝/𝐾 = 100/(2/3 ∗105) = 1.5 x 10-3

Decrease in volume = 𝑒𝑣 𝑉 = 1.5 x 10-3 x (π/4) x 202 x 1000 mm3 = 471mm3

Ques 2. A 50mm cube is subjected to uniform stress of σx=200Mpa. When the change in dimension between two parallel faces in the direction of stress in 0.025mm. determine the change in the volume of the cube. Poisson’s ratio (µ)=0.25

Ques 3. Do you think that a plane stress system give rise to a plane strain system always? Give reasons for your answer. Three principal stresses σ11, σ22,σ33 are acting on the three faces of a cube respectively. Express the resulting three principal strains in terms of the principal stresses.